https://www.cl.cam.ac.uk/projects/raspberrypi/tutorials/temperature/

modprobe w1-gpio

modprobe w1-therm

cd /sys/bus/w1/devices/

cd 28*

/sys/bus/w1/devices/28-000005a8f749# watch " cat w1_slave "

Python

# Open the file that we viewed earlier so that python can see what is in it. Replace the serial  number as before.

tfile = open("/sys/bus/w1/devices/10-000802824e58/w1_slave")

# Read all of the text in the file.

text = tfile.read()

# Close the file now that the text has been read.

tfile.close()

# Split the text with new lines (\n) and select the second line.

secondline = text.split("\n")[1]

# Split the line into words, referring to the spaces, and select the 10th word (counting from 0).

temperaturedata = secondline.split(" ")[9]

# The first two characters are "t=", so get rid of those and convert the temperature from a string to a number.

temperature = float(temperaturedata[2:])

# Put the decimal point in the right place and display it.

temperature = temperature / 1000

print temperature

JE cas se zamyslet….

- po rebootu nezustane modul v jadre / kdo poradi?

- zapsat cas po minute/5 minutach do souboru

  - crontab

  - /var/www/ je root

- soubor dat na web/do DB/

- zpracovat statistiku- kam